# MARIS ONVIF Command Transfer Documentation

| Name | Change Description | Updated On | Version |
| --- | --- | --- | --- |
| Nehama Novick | First document | 29/01/2024 | 1.0.0 |

This document outlines the transferring ONVIF commands from the MARIS server to a camera located at 127.0.0.1 port:2030 using UDP. The MARIS server acts as a mediator between an ONVIF client and the camera, facilitating the execution of various commands for PTZ control.

- Header Message:

This is the structure of the message header, the message header should be at the beginning of every command.

| BYTE | Field Name | Data Type | Field Length (bytes) | Description |
| --- | --- | --- | --- | --- |
| 0 | Sync | Byte | 1 | Intended for checking the validity of a beginning of a message. The word for synchronization is: 0xA5 (165) |
| 1 | Counter | Byte | 1 | 0 - 255 |
| 2 | Command ID | Byte | 1 | Onvif command ID |
| 3 | Check Sum | Byte | 1 | Check Sum on the entire message including the header. |
| 4-5 | Data Length | Short | 2 | Data size only (Not including the header) |

- Continuous Move

Starts continuous movement of the PTZ unit. Movement will continue until a Stop command is sent.

Request Parameters:

- Profile Token: String (required)

- Velocity: Object (required) - The speed of pan, tilt, and zoom.

  - PanTilt: x, y values for pan and tilt.

  - Zoom: x value for zoom.

- Timeout: Duration (optional) - Time after which the movement should stop.

- Absolute Move

Moves the PTZ unit to an absolute destination.

Request Parameters:

  - ProfileToken: String (required)

  - Position: Object (required) - The absolute position for pan, tilt, and zoom.

  - Speed: Object (optional) - The speed of movement.

- Relative Move

Moves the PTZ unit by a relative amount from the current position.

Request Parameters:

  - ProfileToken: String (required)

  - Translation: Object (required) - The relative movement required.

  - Speed: Object (optional) - The speed of movement.

- Set Preset

Sets a new preset at the current position of the PTZ unit.

Request Parameters:

  - ProfileToken: String (required)

  - PresetName: String (optional) - Name of the preset.

  - PresetToken: String (optional) - Token for the preset.

- Goto Preset

Moves the PTZ unit to a specified preset position.

Request Parameters:

  - ProfileToken: String (required)

  - PresetToken: String (required) - Token of the preset to go to.

- Stop

Stops any ongoing movement or zoom.

Request Parameters:

  - ProfileToken: String (required)

  - PanTilt: Boolean (optional) - Stop pan/tilt movements.

  - Zoom: Boolean (optional) - Stop zoom movements.

- All Commands Table:

| Command Name | Command ID |
| --- | --- |
| Get Status | 1 |
| Continuous Move Stop | 2 |
| Absolute Move | 3 |
| Relative Move | 4 |
| Set Preset | 5 |
| Remove Preset | 6 |
| Goto Preset | 7 |
| Go to Home Position | 8 |
| Set Home Position | 9 |
| Set Configuration | 10 |
| Get Preset Tour Options | 11 |
| Create Preset Tour | 12 |
| Modify Preset Tour | 13 |
| Operate Preset Tour | 14 |
| Remove Preset Tour | 15 |
| Send Auxiliary Command | 16 |
| Geo Move | 17 |

- PTZ.h file:

typedef struct

{

uint32	TimeoutFlag 	: 1;				// Indicates whether the field Timeout is valid

uint32  Reserved	 	: 31;

char	ProfileToken[ONVIF_TOKEN_LEN];		// required, A reference to the MediaProfile

onvif_PTZSpeed	Velocity;					// required, A Velocity vector specifying the velocity of pan, tilt and zoom

int		Timeout;							// optional, An optional Timeout parameter, unit is second

} ptz_ContinuousMove_REQ;

typedef struct

{

uint32	PanTiltFlag 	: 1;				// Indicates whether the field PanTilt is valid

uint32	ZoomFlag 		: 1;				// Indicates whether the field Zoom is valid

uint32  Reserved	 	: 30;

char	ProfileToken[ONVIF_TOKEN_LEN];		// required, A reference to the MediaProfile that indicate what should be stopped

BOOL	PanTilt;							// optional, Set true when we want to stop ongoing pan and tilt movements.If PanTilt arguments are not present, this command stops these movements

BOOL	Zoom;								// optional, Set true when we want to stop ongoing zoom movement.If Zoom arguments are not present, this command stops ongoing zoom movement

} ptz_Stop_REQ;

typedef struct

{

uint32	SpeedFlag 		: 1;				// Indicates whether the field Speed is valid

uint32  Reserved	 	: 31;

char	ProfileToken[ONVIF_TOKEN_LEN];		// required, A reference to the MediaProfile

onvif_PTZVector	Position;					// required, A Position vector specifying the absolute target position

onvif_PTZSpeed	Speed;						// optional, An optional Speed

} ptz_AbsoluteMove_REQ;

typedef struct

{

uint32	SpeedFlag 		: 1;				// Indicates whether the field Speed is valid

uint32  Reserved	 	: 31;

char	ProfileToken[ONVIF_TOKEN_LEN];		// required, A reference to the MediaProfile

onvif_PTZVector	Translation;				// required, A positional Translation relative to the current position

onvif_PTZSpeed	Speed;						// optional, An optional Speed parameter

} ptz_RelativeMove_REQ;

typedef struct

{

uint32	PresetTokenFlag : 1;				// Indicates whether the field PresetToken is valid

uint32	PresetNameFlag 	: 1;				// Indicates whether the field PresetName is valid

uint32  Reserved	 	: 30;

char	ProfileToken[ONVIF_TOKEN_LEN];		// required, A reference to the MediaProfile where the operation should take place

char	PresetToken[ONVIF_TOKEN_LEN];		// optional, A requested preset token

char    PresetName[ONVIF_NAME_LEN];			// optional, A requested preset name

} ptz_SetPreset_REQ;

typedef struct

{

char	ProfileToken[ONVIF_TOKEN_LEN];		// required, A reference to the MediaProfile where the operation should take place

char	PresetToken[ONVIF_TOKEN_LEN];		// required, A requested preset token

} ptz_RemovePreset_REQ;

typedef struct

{

uint32	SpeedFlag 		: 1;				// Indicates whether the field Speed is valid

uint32  Reserved	 	: 31;

char	ProfileToken[ONVIF_TOKEN_LEN];		// required, A reference to the MediaProfile where the operation should take place

char	PresetToken[ONVIF_TOKEN_LEN];		// required, A requested preset token

onvif_PTZSpeed	Speed;						// optional, A requested speed.The speed parameter can only be specified when Speed Spaces are available for the PTZ Node

} ptz_GotoPreset_REQ;

typedef struct

{

uint32	SpeedFlag 		: 1;				// Indicates whether the field Speed is valid

uint32  Reserved	 	: 31;

char	ProfileToken[ONVIF_TOKEN_LEN];		// required, A reference to the MediaProfile where the operation should take place

onvif_PTZSpeed	Speed;						// optional, A requested speed.The speed parameter can only be specified when Speed Spaces are available for the PTZ Node

} ptz_GotoHomePosition_REQ;

typedef struct

{

onvif_PTZConfiguration  PTZConfiguration;	// required,

BOOL	ForcePersistence;					// required,

} ptz_SetConfiguration_REQ;

typedef struct

{

char    ProfileToken[ONVIF_TOKEN_LEN];	    // required, Contains the token of an existing media profile the configurations shall be compatible with

} ptz_GetCompatibleConfigurations_REQ;

typedef struct

{

char    ProfileToken[ONVIF_TOKEN_LEN];      // required

} ptz_GetPresetTours_REQ;

typedef struct

{

PresetTourList *  PresetTour;

} ptz_GetPresetTours_RES;

typedef struct

{

char    ProfileToken[ONVIF_TOKEN_LEN];      // required

char    PresetTourToken[ONVIF_TOKEN_LEN];   // required

} ptz_GetPresetTour_REQ;

typedef struct

{

onvif_PresetTour    PresetTour;

} ptz_GetPresetTour_RES;

typedef struct

{

uint32	PresetTourTokenFlag : 1;	        // Indicates whether the field PresetTourToken is valid

uint32  Reserved	 	    : 31;

char    ProfileToken[ONVIF_TOKEN_LEN];      // required

char    PresetTourToken[ONVIF_TOKEN_LEN];   // optional

} ptz_GetPresetTourOptions_REQ;

typedef struct

{

onvif_PTZPresetTourOptions  Options;        // required

} ptz_GetPresetTourOptions_RES;

typedef struct

{

char    ProfileToken[ONVIF_TOKEN_LEN];      // required

} ptz_CreatePresetTour_REQ;

typedef struct

{

char    PresetTourToken[ONVIF_TOKEN_LEN];   // required,

} ptz_CreatePresetTour_RES;

typedef struct

{

char    ProfileToken[ONVIF_TOKEN_LEN];      // required

onvif_PresetTour    PresetTour;             // required

} ptz_ModifyPresetTour_REQ;

typedef struct

{

char    ProfileToken[ONVIF_TOKEN_LEN];      // required

char    PresetTourToken[ONVIF_TOKEN_LEN];   // required

onvif_PTZPresetTourOperation Operation;     // required

} ptz_OperatePresetTour_REQ;

typedef struct

{

char    ProfileToken[ONVIF_TOKEN_LEN];      // required

char    PresetTourToken[ONVIF_TOKEN_LEN];   // required

} ptz_RemovePresetTour_REQ;

typedef struct

{

char    ProfileToken[ONVIF_TOKEN_LEN];      // required,

char    AuxiliaryData[64];                  // required,

} ptz_SendAuxiliaryCommand_REQ;

typedef struct

{

char    AuxiliaryResponse[256];             // required,

} ptz_SendAuxiliaryCommand_RES;

typedef struct

{

uint32	SpeedFlag       : 1;	            // Indicates whether the field Speed is valid

uint32	AreaHeightFlag  : 1;	            // Indicates whether the field AreaHeight is valid

uint32	AreaWidthFlag   : 1;	            // Indicates whether the field AreaWidth is valid

uint32  Reserved        : 29;

char    ProfileToken[ONVIF_TOKEN_LEN];	    // required, A reference to the MediaProfile

onvif_GeoLocation   Target;	                // required, The geolocation of the target position

onvif_PTZSpeed      Speed;	                // optional, An optional Speed

float   AreaHeight;	                        // optional, An optional indication of the height of the target/area

float   AreaWidth;	                        // optional, An optional indication of the width of the target/area

} ptz_GeoMove_REQ;

typedef struct

{

uint32  spaceFlag : 1;                              // Indicates whether the field space is valid

uint32  reserved  : 31;

float	x;										    // required

char    space[256];                                 // optional

} onvif_Vector1D;

typedef struct

{

uint32	PanTiltFlag	: 1;						    // Indicates whether the field PanTilt is valid

uint32	ZoomFlag	: 1;						    // Indicates whether the field Zoom is valid

uint32 	Reserved	: 30;

onvif_Vector 	PanTilt;						    // optional, Pan and tilt position. The x component corresponds to pan and the y component to tilt

onvif_Vector1D	Zoom;							    // optional, A zoom position

} onvif_PTZVector;

typedef struct

{

uint32	PanTiltFlag	: 1;						    // Indicates whether the field PanTilt is valid

uint32	ZoomFlag	: 1;						    // Indicates whether the field Zoom is valid

uint32 	Reserved	: 30;

onvif_Vector 	PanTilt;						    // optional, Pan and tilt speed. The x component corresponds to pan and the y component to tilt. If omitted in a request, the current (if any) PanTilt movement should not be affected

onvif_Vector1D	Zoom;							    // optional, A zoom speed. If omitted in a request, the current (if any) Zoom movement should not be affected

} onvif_PTZSpeed;

typedef struct

{

uint32	PTZPositionFlag	: 1; // Indicates whether the field PTZPosition is valid

uint32 	Reserved		: 31;

char 	Name[ONVIF_NAME_LEN];  // required, A list of preset position name

char 	token[ONVIF_TOKEN_LEN];					    // required

onvif_PTZVector	PTZPosition;					    // optional, A list of preset position

} onvif_PTZPreset;

typedef struct

{

onvif_FloatRange 	XRange;						    // required

onvif_FloatRange	YRange;						    // required

} onvif_PanTiltLimits;

typedef struct

{

onvif_FloatRange 	XRange;						    // required

} onvif_ZoomLimits;